XYTRON Home Security Systems-PS100DG75

Congratulations on your purchase of a Xytron Primary Security Unit.

The Security Unit has been wholly designed and manufactured in South Africa, and the concept has been tried and tested in other products manufactured by us since 1994. Please read these instructions in full before assembling the system.

WARNING!  KEEP OUT OF REACH OF CHILDREN

The Oleoresin Capsicum spray is non-flammable, medically safe and has been approved by the CSIR and the SPCA. The non-toxic pepper fog when sprayed, is instantly effective, non-lethal, and irritates eyes, skin and respiratory system, incapacitating the recipient for up to 35 minutes.

FIRST AID AND DECONTAMINATION

- MOVE PERSON TO A WELL VENTILATED AREA

- RE-ASSURE AND KEEP CALM – THE PERSON CANNOT SEE YOU – TALK CONTINUOUSLY

- WASH AFFECTED PARTS WITH COOL WATER AND A LITTLE DISHWASHING LIQUID (SKIN ONLY, NOT THE EYES)

- USE ONLY COOL WATER TO CLEANSE THE EYES (FILL BASIN WITH WATER AND HAVE THE VICTIM PLACE HIS FACE IN THE WATER AND BLINK EYES RAPIDLY)

- CONTINUE FLUSHING WITH WATER UNTIL SYMPTOMS DISAPPEAR

- NEVER USE OIL, CREAM, SALVE OR OINTMENT – THIS JUST TRAPS THE OLEORESIN CAPSICUM ON THE SKIN

- IF SYMPTOMS PERSIST, OBTAIN IMMEDIATE MEDICAL ATTENTION

Warranties

1The Seller warrants its product to be free from defects in materials and workmanship under normal use for a period of one year from date of manufacture. 2) Unless specifically stated herein, all warranties, express or implied, statutory or otherwise and of whatsoever nature are expressly excluded. 3) The Seller does not guarantee the installation and performance of the product and shall not be responsible for any circumstance resulting from the product’s inability to operate. 4) The Seller’s obligation and liability under this warranty is expressly limited to repairing or replacing, at Seller’s option, any product not meeting the specifications. Returns must include proof of purchase and must be within the warranty period.

NO PERSON is authorized to modify this warranty or make any additional warranties contrary to the warranties as set out herein.

## Disclaimer

The Seller, its employees and/or its agents is absolved and indemnified from: - 1) liability for injury, loss or damage caused by the usage, storage or any other reason whatsoever, whether such injury, loss or damage arose from negligence or otherwise. 2) any claim instituted by any person (including a dependant of such person) for injury (including fatal injury), A  damage or loss of whatever nature, including but without limitation to consequential or special damages, flowing directly or indirectly from the (negligent) usage of the product. 3) claims by any other party caused by defective goods or otherwise arising from the improper, incorrect or otherwise faulty installation or use of the product.

NOTE this system is not a toy, but an aggressive anti-intrusion system

FURTHER NOTE that by purchasing the product, it signifies a person’s acceptance of this agreement.

ALL SYMPTOMS ARE TEMPORARY AND NOT LIFE-THREATENING

The unit is designed to protect your assets, decide what needs protecting.

Getting started

Connect battery connector to open terminal of battery (the system will not work correctly if this connector is not in place), and immediately check the LED on the lid of the box. It must be green. If not green press the White button on the remote. When system is not in operation (i.e. when moving house, or on a journey to where you will use the system for security – holiday home, caravan etc) remove one battery connector. This ensures that the battery does not discharge.

Customizing the Xytron Security System.

The switches allow you to create a security environment of your own choosing.

Switch 5 OFF: The + Trigger then becomes an ARM / DIS-ARM INPUT. When a Positive voltage is supplied to this input then the system will arm. Removing the voltage will dis arm it.

Switch 5 ON: The + Trigger then becomes an extra zone input for external detectors.

Switch 6 OFF: Panic Enabled.

Switch 6 OFF: REMOTE Panic Disabled

Close the front cover of the system; ensuring nozzle is not blocked by the lid.

Plug unit to mains with power lead and transformer provided. The system relies upon permanent power to trickle charge the battery. The battery should have an average life span of two years should the power be permanent. Please note that in the event of the battery being drained totally flat then the battery should be replaced as it would have been damaged and will not give sufficient stand by power

The System:

The system has the ability to be used as a Pepper Spray System, Panic or a conventional Siren alarm.

The four-button remote is a code-hopping transmitter learned to the purchased security unit. This is an extra security feature employed to ensure that the system can only be armed / disarmed by the purchaser. Should you require extra or replacement spray, remotes or batteries in the future, ring 0126573500

Operating the system is simple provided that the following steps are followed:

- After plugging in and switching on at the mains – the LED on the top of the detector will remain Green.

- To Arm the system for Siren ALARM (Siren only) depress the Blue button on the remote, the system will chirp Once. The LED will turn Red and remain Red. When the detector is triggered then the siren only will sound for 30 seconds then reset. The system will still be armed.

- To Disarm the alarm, depress the White button once. The system will chirp Twice and the LED will turn to Green.

- To arm the system for Pepper Spray (recommend this is only performed when leaving the premises vacant), depress the Green button Once. The system will chirp Once and the LED will change from Green to Red and start a 30 second Audible Exit delay before becoming Active and cause the LED to flash Red and Green.

- To Disarm the system, depress the White button on the remote Once only. There will be Two chirps and the LED will turn Green.

- 24Hr PANIC. By depressing the Red button on the remote whilst the system is in the armed or disarmed state will cause the Siren to sound for 30 seconds (no spray). Depressing the Red button again whilst the Panic alarm is sounding will cause the unit to Spray according to the switch settings. To reset the Panic alarm, depress the White button on the remote. The LED will turn Green and the siren will stop.

Lights,  Bells,  Whistles and Buttons

LED STATUS INDICATION:

In the Disarmed State the LED will be GREEN.

In the Siren Armed State the LED will be RED.

In the PEPPER SPRAY Mode the LED will be FLASHING RED AND GREEN.

In the Disarmed state the Status led flashing indicates the amount of activations.

In the Memory mode: Green indicates the onboard detector.

Red indicates the external detectors.

Green/Red indicates the Panic alarm.

Should the alarm / spray have activated in your absence. When you return and de-activate the system with the White button on the remote you will hear two chirps. The LED on the system will indicate weather the system has been activated by flashing either Green or Red or Red/Green.

The LED will start with the amount of activations in Green then Red then fast Red/Green. By counting the amount of Green, Red and Green/RED flashes you will be able to ascertain the amount of activations there were and then work out how many sprays you have left.

Temporary clearing the memory:

After disarming the system and seeing the led flashing you may clear the memory flashes by pressing the White button and holding it for 3 seconds. You will hear the system respond and then the status LED will remain permanently on Green. It is not a problem should you not reset the memory as you will still be able to arm the system again.

Permanently clearing the memory:

This should only be done when the Gas canister is to be replaced. Power down the system. Push dipswitch 1,5 and 6 UP/ON. Press button 2. Reapply the power while pressing Button 2 till you hear the SECOND beep. Remove the power then reset the switches to their original settings. Reapply the power and close the system. The LED will show constantly. The memory is now cleared.

In case of power failure:

The LED behind the system will go out and you will hear a double beep every two minutes until power is restored.

Should the power have been disconnected (power failure, etc.), whilst the system was armed in the spray mode and the battery run down, it will power up in the state it was in when it stopped working.

Remote controls

The two code hopping remote transmitters are factory set to the purchased system. Should a remote transmitter be lost or stolen it is possible to delete the remotes and then re learn them to the system so that the missing remote will not work on the system. Remote transmitters of the same type are available from CPSS.

To remove or add the remotes from the system, follow these instructions:

Button B1 next to the bank of 6 switches above the battery is used to delete or learn the remotes.

To delete all remotes:

Press B1 and hold it down – you will hear the buzzer beep rapidly till it stops and then one long beep. When the buzzer stops sounding you may leave B1. All remotes will now be cleared.

To re-learn all the remotes again:

Press B1, you will hear one chirp – you are now in learn mode. Press the White button on the first remote. You will hear two beeps from the system. Repeat by pressing the White button on the next remote and repeat till the last remote has been learned. Press B1 again to exit learn mode. You will hear a long buzz from the buzzer.

Notes

Should you need to change the canister of OC spray – remove front cover, ensure the steel arm is pushed up and pull the canister towards you from the top. To insert, ensure the steel arm is pushed up and first place the top of canister in to the cavity and then push upwards then push into the base.

Notes for systems without remote transmitters (usually an installed system):

- Ignore instructions regarding code-hopping transmitters.

- Your system will be switched on and off via your chosen means: Key Pad, Key Switch, Existing alarm, SMS COMMUNICATOR etc.

- You will have the panic facility by installing wired panic buttons and wiring them up to the system.

- If you have elected for an LED display box for system status, this will indicate whether a device is armed or not or has been activated.

##### Notes on battery

The Xytron Security Unit is a battery-operated system, and relies upon permanent mains input to trickle charge the battery. In the event of a power cut or failure, the battery will give around 20 hours of usable life, during which, if power is restored the trickle charge will restore the battery life. The battery will decay and go flat if power is removed without removing one battery terminal (installation instruction 3 above). The battery is therefore excluded from the 12-month warranty, but is a replaceable item. This item should be replaced at least every two years. The battery should be tested once every six months.

Notes on actuator:

Should the system be used for demonstration or testing purposes, it is recommended that the time for spray is set for two (2) seconds only, and that a five-minute interval is left between demonstrations as the actuator heats very quickly.

Warranty:

The Xytron Security System has a twelve-month warranty from date of purchase (excluding the rechargeable lead-acid battery, which will go flat if the system is used without power or left uncharged in the box for long periods of time).

It is recommended that the Detector manual be consulted should the system be used in rooms where pets have access.

The aerosol contains 100ml of Pepper Fog, which will last for approximately 15 seconds of spray time. The switch settings that determine the length of spray-time depend on the volume of the room to be purged. A 1-second spray will fog sufficient volume only to make the intruder leave the premises, not to incapacitate.

Aerosols containing demonstration spray has been supplied with your system. This demonstration spray provides a harmless method of determining the effective area of contamination should the real pepper fog be activated. The demonstration aerosol also provides the purchaser with a safe method for periodic testing of the Xytron Security System.

Testing:

Please test the system regularly, at least once a month not less than once every six months.

The best way to test the system is to disconnect the mains and set and trigger the system on the battery only. Should the siren sound change much between the system spraying and the spray resetting then this could be an indication that the battery is not at its best. Should its sound not change much then the battery is in good condition.

It is recommended that a test is performed after severe lightning storms.

Terminals

## 19: OVERRIDE (N/O to 0v)

## 18: AUX (12v OUTPUT/ STATUS)

## 17: Pulsed ON/OFF input. (GSM Arming)

16: SIREN +12v OUTPUT

15: SIREN – 0v OUTPUT

14: BI-COLOUR L.E.D.

13: BI-COLOUR L.E.D.

### 12: TX ****

### 11: DT ***

10: GND -0V

9: PANIC [NO. to 0v]

8: POSITIVE   TRIGGER [N/C.] / ON/OFF INPUT **

7: NEGATIVE TRIGGER [N/C.]

#### 6: GAS MOTOR +12v *

5: GAS MOTOR -   0v

4: DC   0v +

## 3: DC 12v -

2: AC 16v

1: AC 16v

# PASSIVE WIRING OPTIONS

Standard single passive

Siren

* This output switches as long as the gas spray time

** Use this in put to connect other detectors should you want added detection.

*** This input is temporally not used

**** This Output is temporally not used to arm and disarm secondary systems

If you are unsure of any connections do not connect, contact your supplier

# NOTES

-----------------------------------------------------------------------------------------------------------------------------------------------

Serial Number of Unit __________ Date Purchased ________

Name of Purchaser _____________Signature________________

Postal Address _________________________________________

Where Purchased ________________________________________

I have read and understand the contents of the attached instructions.

Xytron accept no responsibility for alien spray used in the system.

Post to Xytron – P O Box 2750, The Reeds. 0158

### Deterrent Security Systems

## Quality products from Xytron

Tel: 0126573500